Assignment 2

Submitted by Athul Jose P

WSU ID 011867566

Executive Summary

Continuation Power Flow

Steps in main.m function

- Initializing 14 bus and importing data

- Making Ybus by calling y_bus_calc.m with taps

- Calculating the scheduled active power (P) and reactive power (Q)

- Finding bus types and assigning to vectors

- Doing Continuation Power Flow by calling CPF.m

Steps in y_bus_calc.m function

- Initializing Ybus with zeros

- Calculating diagonal and off diagonal elements

- Changing terms if tap is present

Steps CPF.m function

- Initializing Voltage magnitude and angles

- Initializing Continuation Parameters

- Finding Initial Values by calling Newton Raphson Function (NR.m)

- Phase 1: Continuation Parameter as Power (lambda)

  - Predictor step with increasing power

  - Corrector step with Newton Raphson method

  - Looping until NR diverges

- Phase 2: Continuation Parameter as Voltage

  - Predictor step with decreasing voltage

  - Corrector step with augmented Jacobian

  - Looping until some percentage (75%) of lambda

- Phase 3: Continuation Parameter as Power (lambda)

  - Predictor step with decreasing power

  - Corrector step with Newton Raphson method

  - Looping until lambda = 0

- Plotting the PV curve

Steps in NR.m function

- Initializing indexes and deltas

- Starting iteration loop which will terminate either if converged or 100 iterations

- Calling dpdq.m for calculating mismatch

- Calling J_calc.m for calculating Jacobian

- Calling fwd_bwd.m for calculating the ΔV and Δδ

- Updating del_V and del_T (magnitude and angle) for next iteration

- Updating the error. Here the error is taken as the maximum of absolute of deltas (ΔV and Δδ)

Steps in dpdq.m function

- Initializing P & Q as zeros

- Calculating P for PV bus and P & Q for PQ bus

Steps in J_calc.m function

- Calculating J1 with loops according to limits (n_bus-1, n_bus-1)

- Calculating J2 with loops according to limits (n_bus-1, n_pq)

- Calculating J3 with loops according to limits (n_pq, n_bus-1)

- Calculating J4 with loops according to limits (n_pq, n_pq)

- Combining all to make J

Steps in fwd_bwd.m function

- Calling LU.m for calculating Lower and Upper elements

- Doing the backward substitution

- Doing the forward substitution

Steps in LU.m function

- Making the Q matrix

- Dividing it into L & U matrices

Results

Statement

I, Athul Jose P, states that all the code written and submitted here is completely done by me. I have not taken any help from others or any online resources.

Athul Jose P

Appendix

% main.m

clc

clear all; close all;

% Initializing 14 bus and importing data

n_bus = 14;

bus_data = importdata('ieee14bus.txt').data;

branch_data = importdata('ieee14branch.txt').data;

% Ybus formation

t = 1; % 0 for without tap, 1 for with tap

Y = y_bus_calc(n_bus,bus_data,branch_data,t);

% Scheduled power calculation

base_MVA = 100;

P_inj = (bus_data(:,8) - bus_data(:,6)) / base_MVA;

Q_inj = (bus_data(:,9) - bus_data(:,7)) / base_MVA;

% Finding bus types

sl_i = find(bus_data(:,3) == 3);

pv_i = find(bus_data(:,3) == 2);

pq_i = find(bus_data(:,3) == 0);

n_pv = length(pv_i);

n_pq = length(pq_i);

% Continuation Power Flow for each load bus

for i = 1:n_pq

n = pq_i(i);

CPF(n,n_bus,bus_data,P_inj,Q_inj,Y,sl_i,pq_i,n_pq);

end

% y_bus_calc.m

function Y = y_bus_calc(N_bs,D_bs,D_br,t)

Y = zeros(N_bs);

% Calculating elements of Ybus

for k = 1:size(D_br,1)

Y(D_br(k,1),D_br(k,1)) = Y(D_br(k,1),D_br(k,1)) + 1/(D_br(k,7) + i*D_br(k,8)) + i*D_br(k,9)/2;

Y(D_br(k,2),D_br(k,2)) = Y(D_br(k,2),D_br(k,2)) + 1/(D_br(k,7) + i*D_br(k,8)) + i*D_br(k,9)/2;

Y(D_br(k,1),D_br(k,2)) = -1/(D_br(k,7) + i*D_br(k,8));

Y(D_br(k,2),D_br(k,1)) = Y(D_br(k,1),D_br(k,2));

end

for k = 1:N_bs

Y(k,k) = Y(k,k) + D_bs(k,14) + i*D_bs(k,15);

end

% adjusting for taps

if(t == 1)

for k = 1:size(D_br,1)

if(D_br(k,15) ~= 0)

t = D_br(k,15);

((t^2) / i*D_br(k,8));

Y(D_br(k,1),D_br(k,1)) = Y(D_br(k,1),D_br(k,1)) + Y(D_br(k,1),D_br(k,2)) - (Y(D_br(k,1),D_br(k,2)))/(t^2);

Y(D_br(k,1),D_br(k,1));

Y(D_br(k,1),D_br(k,2)) = Y(D_br(k,1),D_br(k,2))/t;

Y(D_br(k,2),D_br(k,1)) = Y(D_br(k,1),D_br(k,2));

end

end

end

end

% CPF.m

function CPF(n,n_bus,bus_data,P_inj,Q_inj,Y,sl_i,pq_i,n_pq)

% Initializing Voltage magnitude and angles

V = bus_data(:,11);

V(find(V(:)==0)) = 1;

T = zeros(n_bus,1);

% Initializing Continuation Parameters

P_k = P_inj;

P_k(sl_i) = [];

K = [P_k;Q_inj(pq_i)];

ek1 = [zeros(1,n_bus-1+n_pq) 1];

ek2 = zeros(1,n_bus+n_pq);

ek2_i = find(pq_i == n);

ek2(n_bus-1+ek2_i) = -1;

ek3 = -ek1;

sigma1 = 0.1;

sigma2 = 0.005;

lambda = 0;

i = 1;

% Phase 1: Power as Continuation Parameter

% Initial value from Newton Raphson Method

[V_data,T_data,T1,dvrg] = NR(bus_data,V,T,P_inj*lambda,Q_inj*lambda,n_bus,Y,n_pq,pq_i);

V = V_data(:,size(V_data,2));

T = T_data(:,size(T_data,2));

y(i) = V(n);

x(i) = lambda;

i = i+1;

dvrg = 0;

% Iteration for corrector predictor until divergence

while(dvrg == 0 & i < 100)

% Predictor

theta = T(2:n_bus);

voltage = V(pq_i);

vec = [theta; voltage; lambda];

J = J_calc(bus_data,V,T,Y,n_bus,n_pq,pq_i);

pre = vec + sigma1*((fwd_bwd([J -K; ek1],ek1))');

T = [0; pre(1:n_bus-1)];

for j = 1:n_pq

V(pq_i(j)) = pre(n_bus+j-1);

end

lambda = pre(end);

% Corrector

[V_data,T_data,T1,dvrg] = NR(bus_data,V,T,P_inj*lambda,Q_inj*lambda,n_bus,Y,n_pq,pq_i);

if dvrg == 1

V = prev_V;

T = prev_T;

lambda = prev_lambda;

ph1 = i-1;

else

V = V_data(:,size(V_data,2));

T = T_data(:,size(T_data,2));

prev_V = V;

prev_T = T;

prev_lambda = lambda;

y(i) = V(n);

x(i) = lambda;

i = i+1;

end

end

% Phase 2: Voltage as Continuation Parameter

f_stop = 0.75; % stop factor for second phase

% Increased sigma and stop factor for bus 12 & 13

if (n == 12 | n ==13)

f_stop = 0.98

sigma2 = 0.0005

end

% Iteration for corrector predictor until some percentage of lambda

while(lambda > f_stop*prev_lambda & i < 200)

% Predictor

theta = T(2:n_bus);

voltage = V(pq_i);

vec = [theta; voltage; lambda];

J = J_calc(bus_data,V,T,Y,n_bus,n_pq,pq_i);

pre = vec + sigma2*((fwd_bwd([J -K; ek2],ek1))');

T = [0; pre(1:n_bus-1)];

for j = 1:n_pq

V(pq_i(j)) = pre(n_bus+j-1);

end

lambda = pre(end);

% Corrector

J = J_calc(bus_data,V,T,Y,n_bus,n_pq,pq_i);

[del_P, del_Q] = dpdq_calc(bus_data,V,T,P_inj*lambda,Q_inj*lambda,n_bus,Y);

corr = (fwd_bwd([J -lambda*K; ek2],[del_P del_Q 0]'))';

lambda = lambda + corr(end);

if lambda <= f_stop*prev_lambda

V = prev_V;

T = prev_T;

ph2 = i-1;

break;

else

T = T + [0; corr(1:n_bus-1)];

for j = 1:n_pq

V(pq_i(j)) = V(pq_i(j)) + corr(n_bus+j-1);

end

prev_V = V;

prev_T = T;

y(i) = V(n);

x(i) = lambda;

i = i+1;

end

end

% Phase 3: Power as Continuation Parameter

% Iteration for corrector predictor until negative lambda

while(lambda >= 0 & i < 250)

% Predictor

i;

theta = T(2:n_bus);

voltage = V(pq_i);

vec = [theta; voltage; lambda];

J = J_calc(bus_data,V,T,Y,n_bus,n_pq,pq_i);

pre = vec + sigma1*((fwd_bwd([J -K; ek3],ek1))');

T = [0; pre(1:n_bus-1)];

for j = 1:n_pq

V(pq_i(j)) = pre(n_bus+j-1);

end

lambda = pre(end);

% Corrector

[V_data,T_data,T1,dvrg] = NR(bus_data,V,T,P_inj*lambda,Q_inj*lambda,n_bus,Y,n_pq,pq_i);

V = V_data(:,size(V_data,2));

T = T_data(:,size(T_data,2));

y(i) = V(n);

x(i) = lambda;

i = i+1;

end

% plotting CPF curve

x_label = '\lambda'; % x axis label

y_label = 'Voltage (p.u.)'; % y axis label

legend_name = {'CPF Phase 1','CPF Phase 2','CPF Phase 3'}; % legend names

title_name = ['PV Curve for bus number ' num2str(n)]; % title name

figure('Renderer', 'painters', 'Position', [10 10 800 600])

plot(x(1:ph1),y(1:ph1),'-ob','LineWidth',1.5)

hold on

plot(x(ph1:ph2),y(ph1:ph2),'-ok','LineWidth',1.5)

plot(x(ph2:end),y(ph2:end),'-or','LineWidth',1.5)

xlabel(x_label,'FontSize',18,'FontName','Times New Roman')

ylabel(y_label,'FontSize',18,'FontName','Times New Roman')

legend (legend_name,'Location','northeast')

set(gca,'fontsize',16,'Fontname','Times New Roman','GridAlpha',0.5)

ax = gca

xlim([0 4.5])

title(title_name)

ax.XRuler.Axle.LineWidth = 1.5;

ax.YRuler.Axle.LineWidth = 1.5;

grid

grid minor

saveas(gca,[title_name '.png'])

end

% NR.m

function [V_data,T_data] = NR(bus_data,V,T,P_inj,Q_inj,n_bus,Y,n_pq,pq_i)

% Initializing index

i = 0;

Tol = 1;

del_T = zeros(n_bus,1);

del_V = zeros(n_bus,1);

% Iteration loop

while(Tol > 1e-5 & i < 100)

i = i+1

V = V+del_V;

T = T+del_T;

T_data(:,i) = T;

V_data(:,i) = V;

[del_P, del_Q] = dpdq_calc(bus_data,V,T,P_inj,Q_inj,n_bus,Y);

dpdq = [del_P, del_Q]; % mismatch calculation

J = J_calc(bus_data,V,T,Y,n_bus,n_pq,pq_i); % Jacobian calculation

delta = fwd_bwd(J,dpdq); % finding errors

del_T = [0 delta(1:n_bus-1)]';

for j = 1:n_pq

del_V(pq_i(j)) = delta(n_bus+j-1);

end

Tol = max(abs(delta)) % updating error for convergence

end

end

% dpdq.m

function [del_P, del_Q] = dpdq_calc(bus_data,V,T,P_inj,Q_inj,n_bus,Y)

P = zeros(n_bus,1);

Q = zeros(n_bus,1);

Pi = 1;

Qi = 1;

for i = 1:n_bus

if(bus_data(i,3) ~= 3)

for j = 1:n_bus

P(i) = P(i) + V(i)*V(j)*abs(Y(i,j))*cos(T(i)-T(j)-angle(Y(i,j)));

Q(i) = Q(i) + V(i)*V(j)*abs(Y(i,j))*sin(T(i)-T(j)-angle(Y(i,j)));

end

del_P(Pi) = P_inj(i) - P(i);

Pi = Pi+1;

if(bus_data(i,3) == 0)

del_Q(Qi) = Q_inj(i) - Q(i);

Qi = Qi+1;

end

end

end

end

% J_calc.m

function J = J_calc(bus_data,V,T,Y,n_bus,n_pq,pq_i)

% J1 calculation

J1 = zeros(n_bus-1);

for i = 1:n_bus

for j = 1:n_bus

if(bus_data(i,3) ~=3 & bus_data(j,3) ~=3)

if(i==j)

for k = 1:n_bus

J1(i-1,j-1) = J1(i-1,j-1)+(V(i)*V(k)*abs(Y(i,k))*sin(angle(Y(i,k))-T(i)+T(k)));

end

J1(i-1,j-1) = J1(i-1,j-1) - ((V(i)^2) * (imag(Y(i,i))));

else

J1(i-1,j-1) = -V(i)*V(j)*abs(Y(i,j))*sin(angle(Y(i,j))-T(i)+T(j));

end

end

end

end

J1;

% J2 calculation

J2 = zeros(n_bus-1,n_pq);

for i = 2:n_bus

for j = 1:n_pq

n = pq_i(j);

if(n == i)

for k = 1:n_bus

J2(i-1,j) = J2(i-1,j)+(V(i)*V(k)*abs(Y(i,k))*cos(angle(Y(i,k))-T(i)+T(k)));

end

J2(i-1,j) = (J2(i-1,j) + ((V(i)^2) * (real(Y(i,i)))))/V(i);

else

J2(i-1,j) = V(i)*abs(Y(i,n))*cos(angle(Y(i,n))-T(i)+T(n));

end

end

end

J2;

% J3 calculation

J3 = zeros(n_pq,n_bus-1);

for i = 1:n_pq

n = pq_i(i);

for j = 2:n_bus

if(n==j)

for k = 1:n_bus

J3(i,j-1) = J3(i,j-1)+(V(n)*V(k)*abs(Y(n,k))*cos(angle(Y(n,k))-T(n)+T(k)));

end

J3(i,j-1) = J3(i,j-1) - ((V(n)^2) * (real(Y(n,n))));

else

J3(i,j-1) = -V(n)*V(j)*abs(Y(n,j))*cos(angle(Y(n,j))-T(n)+T(j));

end

end

end

J3;

% J4 calculation

J4 = zeros(n_pq);

for i = 1:n_pq

n1 = pq_i(i);

for j = 1:n_pq

n2 = pq_i(j);

if(n1==n2)

for k = 1:n_bus

J4(i,j) = J4(i,j)+(V(n1)*V(k)*abs(Y(n1,k))*sin(angle(Y(n1,k))-T(n1)+T(k)));

end

J4(i,j) = (- J4(i,j) - ((V(n1)^2) * (imag(Y(n1,n1)))))/V(n1);

else

J4(i,j) = -V(n1)*abs(Y(n1,n2))*sin(angle(Y(n1,n2))-T(n1)+T(n2));

end

end

end

J4;

J = [J1, J2; J3, J4];

end

% fwd_bwd.m

function x = fwd_bwd(A,b)

[L, U] = LU(A);

% Forward Substitution

for k = 1:length(A)

s = 0;

for j = 1:k-1

s = s + (L(k,j)*y(j));

end

y(k) = (b(k) - s) / L(k,k);

end

% Backward Substitution

for k = length(A):-1:1

s = 0;

for j = k+1:length(A)

s = s + (U(k,j)*x(j));

end

x(k) = y(k) - s;

end

end

% LU.m

function [L, U] = LU(a)

Q = zeros(length(a));

for j = 1:length(a)

for k = j:length(a)

s = 0;

for m = 1:j-1

s = s + (Q(k,m)*Q(m,j));

end

Q(k,j) = a(k,j) - s;

end

if j < length(a)

for k = j+1:length(a)

s = 0;

for m = 1:j-1

s = s + (Q(j,m)*Q(m,k));

end

Q(j,k) = (a(j,k) - s) / Q(j,j);

end

end

end

L = tril(Q);

U = Q - L + eye(length(a));

end

% FD.m

function [V_data,T_data] = FD(bus_data,V,T,P_inj,Q_inj,n_bus,Y,n_pq,pq_i)

% Initializing index

B = imag(Y);

B_T =  - B(2:n_bus,2:n_bus);

B_V = - B(pq_i,pq_i);

i = 0;

Tol = 1;

del_T = zeros(n_bus,1);

del_V = zeros(n_bus,1);

% Iteration loop

while(Tol > 1e-5 & i < 100)

i = i+1

V = V+del_V;

T = T+del_T;

T_data(:,i) = T;

V_data(:,i) = V;

[del_P, del_Q] = dpdq_calc(bus_data,V,T,P_inj,Q_inj,n_bus,Y);

P_T = del_P'./V(2:n_bus);

d_T = fwd_bwd(B_T,P_T);

Q_V = del_Q'./V(pq_i);

d_V = fwd_bwd(B_V,Q_V);

del_T = [0 d_T]'; % angle calculation

for j = 1:n_pq

del_V(pq_i(j)) = d_V(j); % magnitude calculation

end

Tol = max(abs([del_T; del_V]));

end

end